Sri Lanka Landslide Susceptibility Mapping

Complete Pipeline — Plain-English Technical Documentation

Preprocessing  →  Phase 1 Analysis  →  Visualisation (Separate)  →  Visualisation (Merged)

# 1. What Does This Pipeline Do? (Big Picture)

This is a landslide risk assessment system for Sri Lanka. It takes raw geographic data elevation maps, soil types, river and fault line locations and produces colour-coded maps showing which parts of each district are most likely to experience a landslide. The entire process runs in four ordered steps:

- preprocessing.py  — Cleans and prepares all the raw data.

- phase1_susceptibility.py  — Calculates a risk score for every point on the map.

- visualization_seperate.py  — Creates a detailed map image for each individual district.

- visualization_merged.py  — Creates combined national-level maps and statistics.

# 2. Key Terms Explained

## 2.1 Geographic Terms

| Term | Plain-English Meaning |
| --- | --- |
| DEM | Digital Elevation Model. A raster (grid) file where every cell stores the height (altitude) of that piece of ground in metres. The code uses the SRTM 30 m dataset — meaning each grid cell covers a 30 × 30 metre square. |
| Raster / .tif | A grid of cells, like a spreadsheet of numbers, where every cell represents a geographic location. A '.tif' (GeoTIFF) file is the standard format for storing these grids with location information attached. |
| Shapefile / .shp | A vector file that stores geographic boundaries as polygons (areas), lines, or points — e.g., district borders, rivers, fault lines. |
| CRS / EPSG | Coordinate Reference System. The mathematical rules that map 3D Earth onto a flat grid. EPSG:32644 is the UTM Zone 44N projection, measuring distances in metres. EPSG:4326 (WGS84) uses latitude/longitude degrees. |
| Reprojection | Converting data from one coordinate system to another — like translating a document from one language to another so all layers use the same units. |
| Clip / Mask | Cutting a large map layer down to exactly the shape of a district, like using a cookie cutter. |
| Pixel / Cell | One grid square in a raster file. At 30 m resolution, each pixel covers 900 square metres of land. |
| NaN / nodata | Not a Number. Cells outside the study area (e.g. ocean) are marked NaN so the maths ignores them. The code stores these as −9999.0 in saved files. |
| District polygon | The boundary shape of an administrative district (like Kegalle or Kandy), stored as a shapefile. |
| Resample | Changing the resolution of a raster — e.g. from 90 m cells to 30 m cells — so all layers are the same grid size and can be overlaid precisely. |

## 2.2 Terrain & Topographic Terms

| Term | Plain-English Meaning |
| --- | --- |
| Elevation | Simply the height of the land above sea level, in metres. |
| Slope | How steep the ground is, measured in degrees. 0° = flat; 90° = vertical cliff. Steeper slopes are more likely to slide. |
| Aspect | The compass direction that a slope faces (north, south, east, west). Expressed as 0°–360°. |
| Plan Curvature | Whether a slope curves sideways (left–right). Negative = concave, water converges there. Positive = convex, water diverges. |
| Profile Curvature | Whether a slope curves uphill-to-downhill. Negative = concave-down (slope accelerates). Positive = convex-down (slope decelerates). |
| TWI | Topographic Wetness Index. A number estimating how wet the soil is based purely on shape of the land. Higher TWI = wetter = more risk. |
| TRI | Terrain Ruggedness Index. Measures how rough/uneven the terrain is around each point. High TRI = rugged = less stable. |
| SPI | Stream Power Index. Estimates how much erosion a water flow can cause at each point. Higher SPI = more erosive power. |
| Pixel size (ps) | The width of one grid cell in metres (30 m). Used in slope and curvature maths. |

## 2.3 Hydrological Terms

| Term | Plain-English Meaning |
| --- | --- |
| Flow Direction | For each cell, which neighbouring cell water flows into (downhill direction). Encoded as numbers: 1=East, 2=SE, 4=South, 8=SW, 16=W, 32=NW, 64=N, 128=NE. |
| Flow Accumulation | How many cells drain through each cell. A cell with high accumulation is effectively in a stream channel. |
| MFD | Multiple Flow Direction — a method that splits flow between multiple downhill neighbours instead of just the steepest one, giving more realistic water routing. |
| Stream Threshold (1000) | The minimum flow accumulation number used to identify a 'stream'. Cells with accumulation ≥ 1000 are treated as stream channels. |
| Drainage Density | How many kilometres of stream channels exist per square kilometre of land in a local area. Higher = more dissected, more erosion potential. |
| Fill Depressions | Flat or sunken spots ('pits') in DEM data cause flow direction calculations to fail. 'Filling' raises them slightly so water can flow out. The epsilon=True flag adds a tiny gradient to avoid infinite loops. |

## 2.4 Soil Terms

| Term | Plain-English Meaning |
| --- | --- |
| Clay / Sand / Silt | The three particle sizes that make up soil texture. Clay = finest; Silt = medium; Sand = coarsest. The proportions determine how much water soil holds and how it fails. |
| SoilGrids | A global soil dataset. Values are stored in grams per kilogram (g/kg); dividing by 10 converts them to percentages. |
| HSG | Hydrologic Soil Group. A standard 4-class system (A, B, C, D) grouping soils by drainage speed: A = fast-draining sandy soils; D = slow-draining clay soils. Used in the SINMAP stability calculation. |
| SOIL_SCALE = 10 | The raw soil data is in g/kg; dividing by 10 gives %. |

## 2.5 Land Use / Land Cover Terms

| Term | Plain-English Meaning |
| --- | --- |
| LULC | Land Use and Land Cover. Categories like forest, bare rock, paddy fields, built-up areas. Each category is assigned a risk score based on how well vegetation holds the soil. |
| DN (Digital Number) | The numeric code stored in the LULC shapefile for each land cover class (e.g. 1 = Bare rock, 2 = Forest). |
| Root Cohesion | Extra soil strength provided by tree roots. Forests have higher root cohesion (8 kPa) than bare land (0 kPa). |
| Rasterize | Converting a vector shapefile (polygons) into a raster grid so it can be overlaid with DEM layers. |

## 2.6 Stability & Risk Terms

| Term | Plain-English Meaning |
| --- | --- |
| SINMAP FS | Factor of Safety the ratio of forces holding soil in place to forces pulling it downhill. FS < 1.0 means the slope is theoretically unstable. FS = 1.5 is generally considered safe. |
| Cohesion (C) | Stickiness of soil particles how well they cling together even without friction. |
| Friction angle (phi) | The internal friction of soil. Higher = steeper slopes can remain stable. |
| Soil bulk density (rho) | How dense the soil is (kg/m³). Heavier soils exert more gravitational force. |
| Saturation ratio (m) | What fraction of the soil pore space is filled with water (0 = dry, 1 = fully saturated). |
| MCE | Multi-Criteria Evaluation. Combining multiple factors (slope, TWI, soil, etc.) into a single score using a weighted formula. |
| AHP Weights | Analytic Hierarchy Process. A peer-reviewed method for deciding how much influence each factor should have. All weights must sum to 1.0 (100%). |
| Susceptibility Class | The final 1–5 risk category: 1=Very Low, 2=Low, 3=Moderate, 4=High, 5=Very High. |
| Adaptive Thresholds | Instead of fixed number ranges for every district, the system calculates thresholds from that district's own data distribution (percentiles), so flat coastal districts and steep mountain districts are evaluated fairly. |
| Percentile (P20, P40...) | The value below which a certain percentage of pixels fall. P20 of TWI means 20% of pixels have TWI below this value. |
| Choropleth | A map where each administrative region is shaded according to a statistic (e.g. mean risk score). |

## 2.7 Programming / Code Terms

| Term | Plain-English Meaning |
| --- | --- |
| import | Loads an external library/tool. Like plugging in an appliance before using it. |
| numpy (np) | The core numerical computing library. An 'array' is a grid of numbers that can be processed all at once without loops — very fast. |
| geopandas (gpd) | Handles geographic vector data (shapefiles). Like a spreadsheet where one column is a geometry/shape. |
| rasterio | Reads and writes raster files (.tif). Handles coordinate systems, pixel sizes, etc. |
| richdem (rd) | A library specialised in terrain analysis: flow direction, flow accumulation, depression filling. |
| scipy.ndimage | Scientific image/array processing: used here for neighbourhood smoothing (uniform_filter) and distance transforms. |
| tqdm | Displays animated progress bars in the terminal so you can see how far the processing has progressed. |
| matplotlib | Python's main plotting library. Used to draw the final map images. |
| geopandas.dissolve | Merges many village polygons into a single district polygon. |
| np.nan / np.nanmean | Ignores NaN cells when computing statistics like mean. |
| os.makedirs | Creates a folder (and any missing parent folders) on disk. |
| GeoDataFrame (gdf) | A table of geographic features, each with a shape (geometry) and data columns. |
| transform | A mathematical description of how raster pixels map to real-world coordinates (origin, pixel size, rotation). |

# 3. Script 1: preprocessing.py

Purpose:  Reads all raw input files, reprojects and aligns them to the same coordinate system and resolution, then extracts and saves every geographic feature needed for Phase 1 district by district.

## 3.1 Configuration Block

The first thing the script does is define paths and settings. Think of this as filling out a form before starting work.

Key settings defined: DEM_PATH — Path to the elevation raster for all of Sri Lanka (SRTM 30 m resolution). VILLAGE_SHP — Shapefile of all village boundaries, including a column (adm2_name) with district names. FAULT_SHP / RIVER_SHP — Shapefiles of fault lines and rivers. SOIL_FILES — Paths to three soil texture rasters: clay, sand, and silt. TARGET_CRS = EPSG:32644 — All data will be converted to this metric coordinate system (metres). TARGET_RES = 30 — All output rasters will have 30 m × 30 m cells. STREAM_THRESHOLD = Flow accumulation cells above this value are treated as streams.

## 3.2 District Selector (select_districts)

When the script starts, it asks the user which districts to process. The user can type 'all', type a single name like 'Kegalle', or type several names separated by commas. The script validates the names against the actual district list and confirms the choice before continuing.

## 3.3 Reprojection Helpers

### reproject_dem

Takes the DEM file (which may use latitude/longitude degrees) and converts it to the metric coordinate system EPSG:32644. The result is saved as a new file ending in _metric.tif. If the file already exists, it is reused (no repeated work).

The bilinear resampling method used here is like smooth interpolation — instead of using the nearest pixel, it averages nearby pixels to avoid blocky staircase patterns.

### reproject_raster

Same idea as above but also lets the caller specify the target resolution. Used for the soil rasters, which may originally have different cell sizes.

## 3.4 Clip and Save Helpers

### clip_dem_to_polygon

Cuts the reprojected DEM down to the exact boundary of one district and returns the clipped array plus metadata. Any cells outside the district boundary are set to NaN.

### clip_and_save_raster

The same as above but immediately writes the clipped raster to disk as a compressed GeoTIFF. Used for soil layers.

### save_tif

A simple helper that writes any computed array to disk as a .tif file, replacing NaN with −9999.0 (the standard nodata marker for GeoTIFF).

### resample_raster_inplace

Rewrites an existing raster file at a new resolution. Used in Stage 5 to standardise everything to exactly 30 m. It writes to a temporary file first, then renames it — this prevents data loss if the process is interrupted.

## 3.5 Stage 1 — Topographic Features (run_topographic)

For each selected district, this stage reads the clipped DEM and computes eight terrain layers. Each is saved as a separate .tif file.

| Term | Plain-English Meaning |
| --- | --- |
| Elevation | Simply copies the DEM values — the height of the land at each pixel. |
| Slope | Calculates steepness by comparing each cell's height to its left/right and up/down neighbours using the Pythagoras-based formula. The gradient is converted from rise/run to degrees. |
| Aspect | Finds the direction the slope faces using the arctan2 function (a maths function that gives angles from X/Y components of the gradient). |
| Plan Curvature | Measures sideways bending of the terrain. Computed using second derivatives — i.e. how quickly the slope direction is changing horizontally. |
| Profile Curvature | Measures up/down-slope bending. Computed using second derivatives in the downslope direction. |
| TWI | Fills depressions, computes flow accumulation with D8 method, then applies the formula: ln(upslope_area / tan(slope)). The richdem library handles the depression filling and accumulation. |
| TRI | For each cell, computes the square root of the sum of squared elevation differences to all 8 neighbours. High values = rough terrain. |
| SPI | Flow accumulation multiplied by tan(slope). High values indicate steep slopes carrying large upslope areas — high erosion power. |

The progress bar (tqdm) shows two levels: one for districts (green) and one for individual features (cyan).

## 3.6 Stage 2 — Hydrological Features (run_hydrological)

Computes three water-flow layers for each district.

### compute_flow_direction_vectorized

After filling depressions, compares each cell's elevation to its 8 neighbours. The neighbour with the steepest downhill gradient 'wins', and the cell is assigned its direction code. Diagonal neighbours use a slightly longer distance (√2 × pixel_size) in the slope formula.

### compute_flow_accumulation_mfd

Processes cells from highest to lowest elevation. For each cell, it distributes its accumulated water to downhill neighbours in proportion to their slope gradient raised to power p=1.1. This multiple-flow method is more realistic than D8 (which forces all flow into a single neighbour).

### compute_drainage_density

First creates a binary stream mask (cells where accumulation ≥ STREAM_THRESHOLD = 1, others = 0). Then for each cell, sums stream lengths in a 21×21 pixel window around it and divides by the window area in km². The result is drainage density in km/km².

## 3.7 Stage 3 — Soil Features (run_soil)

For each district, clips the three soil rasters (clay, sand, silt) to the district boundary and saves them. No computation is done here — that happens in Phase 1.

## 3.8 Stage 4 — Vector Features (run_vector)

Computes distance-from-fault and distance-from-river rasters for each district.

### clip_vector

Clips the fault/river line shapefile to the district polygon, with a 2,000 m buffer to capture features just outside the boundary.

### compute_distance_raster

First rasterizes the fault/river lines — burns them onto the raster grid as 1s, with 0s elsewhere. Then uses distance_transform_edt (Euclidean Distance Transform) which computes, for every 0-cell, the straight-line distance (in pixels) to the nearest 1-cell. Multiplying by pixel_size converts pixels to metres.

## 3.9 Stage 5 — Resample All Outputs (run_resample_all)

Walks through all output folders, collects every .tif file, and re-saves each one at exactly 30 m resolution in the target CRS. This ensures all layers are perfectly aligned before Phase 1 stacks them together.

## 3.10 Main Function

Ties everything together in order:

- Load the village shapefile and ask the user to select districts.

- Dissolve village polygons into district-level polygons and reproject to EPSG:32644.

- Reproject the DEM to metric coordinates.

- Reproject all soil rasters.

- Load fault and river shapefiles.

- Run all five stages in sequence.

# 4. Script 2: phase1_susceptibility.py

Purpose:  Reads all preprocessed rasters, calculates the SINMAP Factor of Safety, applies an AHP-weighted multi-criteria evaluation, and produces a 1–5 susceptibility class map and shapefile for each district.

## 4.1 AHP Base Weights

These ten numbers tell the model how important each factor is. They must all add up to exactly 1.0 (100%). The values come from published peer-reviewed research:

| Term | Plain-English Meaning |
| --- | --- |
| slope = 0.22 | 22% — the single most important factor. Steeper slopes fail more easily. |
| twi = 0.16 | 16% — wetness drives saturation, the main trigger for shallow landslides. |
| curvature = 0.12 | 12% — concave slopes concentrate water and stress. |
| drainage_density = 0.10 | 10% — highly dissected terrain has more potential failure surfaces. |
| spi = 0.09 | 9% — stream power drives undercutting at the base of slopes. |
| dist_to_fault = 0.08 | 8% — fault zones have fractured, weakened rock. |
| dist_to_river = 0.07 | 7% — river banks are undercut and saturated. |
| lulc = 0.06 | 6% — vegetation provides root cohesion; bare land does not. |
| fs = 0.06 | 6% — the SINMAP stability index provides a physics-based check. |
| tri = 0.04 | 4% — ruggedness is a secondary terrain factor. |

## 4.2 Dynamic Weight Adjustment (adjust_weights_for_district)

The weights above are defaults. Two automatic adjustments happen before processing each district:

- No fault data:  If no fault shapefile data exists for the district, the 0.08 fault weight is split equally between TWI (+0.04) and dist_to_river (+0.04). The total still sums to 1.0.

- Flat terrain:  If more than 85% of the district's slope pixels are below 10°, the district is considered too flat for slope-driven landslides. Slope weight is reduced by 0.048 and SPI by 0.032; TWI and drainage density each gain 0.04. This helps correctly assess coastal flood-plain districts like Gampaha.

## 4.3 Fixed Thresholds

Some factors have universally agreed physical breakpoints that do not vary by location:

| Term | Plain-English Meaning |
| --- | --- |
| Slope (Dai & Lee 2002) | 0–10° = class 1; 10–20° = 2; 20–30° = 3; 30–45° = 4; >45° = 5 (Very High) |
| Dist. to Fault (Yalcin 2008) | <500 m = 5; 500–1000 m = 4; 1–2 km = 3; 2–5 km = 2; >5 km = 1 |
| Factor of Safety (Pack 1998) | FS < 1.0 = class 5; 1.0–1.25 = 4; 1.25–1.5 = 3; 1.5–2.0 = 3; >2.0 = 3 |

## 4.4 Adaptive Thresholds (adaptive_thresholds)

For factors without universal breakpoints (TWI, TRI, SPI, Drainage Density, Curvature, Distance to River), thresholds are calculated from the district's own pixel distribution:

- Extract all valid (non-NaN) pixel values for the factor.

- Compute the 20th, 40th, 60th, and 80th percentile values (P20, P40, P60, P80).

- Use these four values as the class boundaries, creating five equal-frequency bins.

- high_risk_direction:  For TWI, TRI, SPI, Drainage Density — large values = high risk (class 5 at the top). For Curvature and Distance to River — small values = high risk (class 5 at the bottom).

The log_thresholds function then prints a bar-chart breakdown of how many pixels fall in each class, so the user can verify the classification is sensible.

## 4.5 SINMAP Factor of Safety (compute_fs)

This is a physics-based slope stability formula. It asks: 'Is the soil strong enough to resist sliding?'

The formula used is the infinite-slope stability model:

FS = (Cohesion + cos²(slope) × (soil_density − m × water_density) × gravity × depth × tan(friction_angle)) ÷ (soil_density × gravity × depth × sin(slope) × cos(slope)) Where m (saturation) is estimated from normalised TWI, capped at 0.30 (static dry conditions).

Soil parameters (cohesion, friction angle, density, depth) are looked up from the HSG class (A/B/C/D). Root cohesion from vegetation (LULC) is added to soil cohesion.

The result is clipped to 0–10 to avoid unrealistic outliers. FS < 1.0 means the slope is theoretically failing.

## 4.6 Soil HSG Classification

From the three soil layers (clay %, sand %, silt %) the code assigns each pixel to a Hydrologic Soil Group:

| Term | Plain-English Meaning |
| --- | --- |
| Class A (sandy) | Sand > 70% AND Clay < 10% → fast-draining, low risk |
| Class B (loam) | Sand 50–70% OR Silt > 50% with Clay < 20% |
| Class C (clay-loam) | Clay 20–40% with Sand < 50% |
| Class D (clay) | Clay ≥ 40% → slow-draining, high water retention, highest risk |

If soil data is missing for a pixel (NaN), it is filled with the most common class in that district (majority fill). If all soil data is missing, class B (Loam) is used as a safe default.

## 4.7 Classification (classify)

This function converts continuous numbers (e.g. slope = 27.4°) into integer classes (1–5). It uses np.digitize — a single-pass binning operation that is much faster than checking each pixel one by one with multiple if statements.

## 4.8 MCE Weighted Sum

After all ten factors have been classified to 1–5 scores, the final susceptibility score is:

MCE_score = Σ (weight_i × class_score_i) for i in all factors Example: 0.22 × slope_class + 0.16 × twi_class + 0.12 × curvature_class + …

The result is a continuous score between 1.0 (Very Low) and 5.0 (Very High). Cells where the slope layer is NaN (outside the district) are set to NaN.

## 4.9 Final Classification and Shapefile (classify_mce, to_shapefile)

The continuous MCE score is binned into the five named risk classes:

| Term | Plain-English Meaning |
| --- | --- |
| < 1.5 | Class 1 — Very Low |
| 1.5 – 2.5 | Class 2 — Low |
| 2.5 – 3.5 | Class 3 — Moderate |
| 3.5 – 4.5 | Class 4 — High |
| ≥ 4.5 | Class 5 — Very High |

The to_shapefile function converts the raster class grid back into vector polygons using rasterio.features.shapes, then dissolves adjacent pixels of the same class into larger polygons and calculates their area in km². The result is a standard .shp file ready for use in any GIS application.

## 4.10 Outputs per District

| Term | Plain-English Meaning |
| --- | --- |
| _factor_of_safety.tif | Continuous SINMAP FS values (0–10) |
| _susceptibility_score.tif | Continuous MCE score (1.0–5.0) |
| _susceptibility_class.tif | Integer class raster (1, 2, 3, 4, 5) |
| _susceptibility.shp | Vector polygons of dissolved risk classes with area_km2 field |

# 5. Script 3: visualization_seperate.py

Purpose:  Reads the Phase 1 outputs for each district and produces one detailed PNG image showing four panels of information.

## 5.1 Colour Scheme

A consistent five-colour scale is used throughout:

| Term | Plain-English Meaning |
| --- | --- |
| #2ecc71 (Green) | Class 1 — Very Low risk |
| #a8d8a8 (Light Green) | Class 2 — Low risk |
| #f39c12 (Orange) | Class 3 — Moderate risk |
| #e67e22 (Dark Orange) | Class 4 — High risk |
| #c0392b (Dark Red) | Class 5 — Very High risk |

## 5.2 Reprojection to WGS84 (reproject_to_wgs84)

The stored rasters use EPSG:32644 (metres). For map display purposes they are reprojected to WGS84 (latitude/longitude), which is what web maps and standard GIS tools expect.

## 5.3 Array to PNG Conversion (array_to_png_base64)

Converts a numeric raster array into a colour image. The steps are: normalise values to 0–1, look up the colour from the colourmap, set transparency (alpha=0.75), and encode as a PNG file embedded in base64 text. This format is used for overlay layers.

## 5.4 Static Figure (make_static_figure) — 6 Panels

The figure has a dark background (#0d1117) and is laid out as 2 rows × 3 columns:

| Term | Plain-English Meaning |
| --- | --- |
| Panel A — Susceptibility Class | Displays the integer 1–5 class raster using the five risk colours. A legend is shown in the lower left. |
| Panel B — Susceptibility Score | Displays the continuous 1.0–5.0 MCE score using a red-yellow-green colour ramp (RdYlGn_r reversed). A colour bar on the right shows the scale. |
| Panel C — Factor of Safety | Displays FS values using green-yellow-red (green = stable, red = unstable). Values capped at 0–4 for visual contrast. |
| Panel D — Area by Risk Class (Pie) | A pie chart showing what percentage of the district's area falls in each risk class. Slices below 2% are not labelled. |
| Panel E — AHP Factor Weights (Bar) | A horizontal bar chart of the 10 AHP weights as percentages. Bars are colour-coded: red for weights ≥15%, orange for ≥9%, etc. |
| Panel F — Summary Statistics (Table) | Min, Max, Mean, Median of the score and FS, percentage of pixels with FS < 1.0, and percentage of pixels in High or Very High classes. |

## 5.5 Top-10 Hotspot Finder (get_top10_hotspots)

Finds the 10 pixels with the highest susceptibility score across the district. To avoid returning 10 points all clustered in the same small area, a minimum spacing rule (30 pixels ≈ 900 m) is enforced: once a pixel is selected, no pixel within 30 rows and 30 columns can be selected again. Each hotspot is returned as a dictionary with its latitude, longitude, score, and FS value.

## 5.6 Main Function Flow

- Load the village shapefile.

- Ask the user to select districts.

- For each district: verify all three output files exist.

- Load the three rasters and trim them to the same shape.

- Print a console summary of class percentages.

- Call make_static_figure and save the PNG.

## 5.7 Output

<District>_susceptibility_map.png — a 20×14 inch PNG at 150 DPI showing all six panels with a title and method footnote.

# 6. Script 4: visualization_merged.py

Purpose:  Automatically discovers all processed districts, aggregates their statistics, and produces two national-level summary images.

## 6.1 Automatic District Discovery (discover_districts)

Unlike the other scripts, this one does not ask the user to pick districts. It scans the output folder and includes any district sub-folder that contains all three required .tif files. This makes it fully automatic once the Phase 1 outputs exist.

## 6.2 Statistics Collection (build_stats)

For each district, the full-resolution rasters are loaded, statistics are computed, and then the arrays are immediately deleted from memory. This district-by-district approach prevents running out of RAM when processing all 25 Sri Lankan districts simultaneously. Statistics collected per district:

- Mean and median susceptibility score

- Mean Factor of Safety

- Dominant risk class (the class with the most pixels)

- Percentage of pixels in each of the 5 classes

- Percentage of pixels classified as High + Very High combined

- Percentage of pixels with FS < 1.0 (theoretically unstable)

- A random 1-in-50 sample of score values (for the national curve chart)

## 6.3 Image 1 — Geographic Choropleth Maps (make_choropleth_maps)

Produces a 22×20 inch PNG with four map panels of Sri Lanka, each shaded by a different metric. All maps are aligned to WGS84 and show the full island extent (longitude 79.5°–82.0°, latitude 5.85°–9.9°).

| Term | Plain-English Meaning |
| --- | --- |
| Map A — Mean Susceptibility Score | Each district is filled with a colour from the RdYlGn_r scale (green=safe, red=dangerous) corresponding to its mean MCE score. A colour bar on the right shows the scale. |
| Map B — Dominant Risk Class | Each district is filled with one of the five categorical colours based on which risk class covers the most area in that district. |
| Map C — Mean Factor of Safety | Each district coloured green→red by its mean FS value. Districts with mean FS < 1.0 (rare) are additionally marked with a red star. |
| Map D — High + Very High Area % | Districts shaded from white to dark red showing what fraction of their area is High or Very High risk. |

District names are printed at the centroid of each district polygon using add_district_labels. Long names are wrapped to two lines. A semi-transparent dark background behind each label improves legibility.

## 6.4 Image 2 — Cumulative Risk Analysis (make_risk_analysis)

Produces a 26×20 inch PNG with seven analytical panels:

| Term | Plain-English Meaning |
| --- | --- |
| Panel A — Top-10 Highest-Risk Districts | Horizontal bar chart, sorted descending by mean score. Bars are coloured by risk level. Rank medals (gold/silver/bronze) mark the top 3. Reference lines at score = 3 and 4 are shown. |
| Panel B — All Districts by Mean FS | Horizontal bar chart of every district sorted by FS ascending (most unstable at top). Reference lines at FS = 1.0 and 1.5. |
| Panel C — Risk Class Heatmap | A grid where rows are the 5 risk classes and columns are all districts (sorted by mean score). Cell colour and number show what % of each district falls in each class. |
| Panel D — Cumulative Area-at-Risk Curve | A national curve: x-axis = cumulative % of all pixels; y-axis = susceptibility score. The curve shows what fraction of Sri Lanka's land area exceeds each risk level. Reference lines at score = 3 (Moderate) and 4 (High) are annotated. |
| Panel E — Mean Score vs High-Risk Area Scatter | Each dot is one district. X-axis = mean score; Y-axis = % High+VH area. A trend line is fitted. The 7 highest-risk districts are labelled. |
| Panel F — National Risk Composition Pie | A weighted pie chart showing the percentage of all land area (across all processed districts) in each risk class. |
| Panel G — District Risk League Table | A ranked table with columns: Rank, District, Mean Score, Mean FS, High+VH%. Top 3 are gold/silver/bronze. Background shading alternates for readability. |

## 6.5 Console Summary (print_console_summary)

Prints a text summary to the terminal: total districts analysed, total pixels, national High+VH percentage, the top 10 highest-risk districts, and the 5 most stable districts.

## 6.6 Outputs

| Term | Plain-English Meaning |
| --- | --- |
| cumulative_susceptibility_overview.png | The four geographic choropleth maps of Sri Lanka. |
| cumulative_risk_analysis.png | The seven statistical analysis panels. |

# 7. How to Run the Complete Pipeline

The four scripts must be run in this exact order. Each script depends on the outputs of the previous one.

- Step 1 — Preprocessing:

  - Run python preprocessing.py

  - When prompted, select the districts you want to process.

  - The script will run all five stages and print progress bars.

  - Outputs: raster .tif files in four sub-folders (Topo_Output, Hydro_Output, Soil_Output, Vector_Output).

- Step 2 — Phase 1 Analysis:

  - Run python phase1_susceptibility.py

  - Select the same districts (or a subset).

  - The script reads all preprocessed layers and produces susceptibility maps.

  - Outputs: _susceptibility_class.tif, _susceptibility_score.tif, _factor_of_safety.tif, _susceptibility.shp per district.

- Step 3 — Individual Visualisations:

  - Run python visualization_seperate.py

  - Select districts.

  - Outputs: one _susceptibility_map.png per district.

- Step 4 — National Visualisation:

  - Run python visualization_merged.py

  - No district selection needed — runs automatically on all available data.

  - Outputs: cumulative_susceptibility_overview.png and cumulative_risk_analysis.png.

# 8. Full Output File Reference

This table lists every file the pipeline produces, organised by script.

| Script | File Pattern | Description |
| --- | --- | --- |
| preprocessing.py | <District>_elevation.tif | Raw elevation raster clipped to district |
| preprocessing.py | <District>_slope.tif | Slope in degrees |
| preprocessing.py | <District>_aspect.tif | Slope aspect in degrees |
| preprocessing.py | <District>_plan_curvature.tif | Plan curvature |
| preprocessing.py | <District>_profile_curvature.tif | Profile curvature |
| preprocessing.py | <District>_twi.tif | Topographic Wetness Index |
| preprocessing.py | <District>_tri.tif | Terrain Ruggedness Index |
| preprocessing.py | <District>_spi.tif | Stream Power Index |
| preprocessing.py | <District>_flow_direction.tif | D8 flow direction codes |
| preprocessing.py | <District>_flow_accumulation_mfd.tif | MFD flow accumulation |
| preprocessing.py | <District>_drainage_density.tif | Drainage density (km/km²) |
| preprocessing.py | <District>_clay/sand/silt.tif | Soil texture percentages |
| preprocessing.py | <District>_dist_to_fault.tif | Distance to nearest fault (m) |
| preprocessing.py | <District>_dist_to_river.tif | Distance to nearest river (m) |
| phase1.py | <District>_factor_of_safety.tif | SINMAP FS (0–10) |
| phase1.py | <District>_susceptibility_score.tif | MCE score (1.0–5.0) |
| phase1.py | <District>_susceptibility_class.tif | Integer class raster (1–5) |
| phase1.py | <District>_susceptibility.shp | Vector polygons with area_km2 |
| viz_separate.py | <District>_susceptibility_map.png | 6-panel district summary image |
| viz_merged.py | cumulative_susceptibility_overview.png | 4-panel national choropleth maps |
| viz_merged.py | cumulative_risk_analysis.png | 7-panel national analysis charts |

# 9. Scientific References

The methodology is grounded in published peer-reviewed research. Key references cited in the code:

- Yalcin, A. (2008). GIS-based landslide susceptibility mapping using analytical hierarchy process and bivariate statistics in Ardesen (Turkey). Catena, 72(1), 1–12.

- Pradhan, B. & Lee, S. (2010). Regional landslide susceptibility analysis using back-propagation neural network model at Cameron Highland, Malaysia. Landslides, 7(1), 13–30.

- Pourghasemi, H.R. et al. (2012). Application of fuzzy logic and analytical hierarchy process (AHP) to landslide susceptibility mapping at Haraz watershed, Iran. Natural Hazards, 63, 965–996.

- Pack, R.T., Tarboton, D.G. & Goodwin, C.N. (1998). The SINMAP approach to terrain stability mapping. 8th Congress of the International Association of Engineering Geology.

- Dai, F.C. & Lee, C.F. (2002). Landslide characteristics and slope instability modelling using GIS, Lantau Island, Hong Kong. Geomorphology, 42(3–4), 213–228.

- Bui, D.T. et al. (2016). Landslide susceptibility assessment in the Hoa Binh province of Vietnam. Remote Sensing, 8(11), 901.

- Hong, H. et al. (2017). Landslide susceptibility assessment in Lianhua County (China). Science of the Total Environment, 582, 756–769.

9. Key terms with their scientific references:

- Slope (Steepness of terrain) Dai, F.C. & Lee, C.F. (2002). Landslide characteristics and slope instability modelling using GIS, Lantau Island, Hong Kong. Geomorphology, 42(3–4), 213–228. Fixed slope thresholds: 0–10°, 10–20°, 20–30°, 30–45°, >45°

- AHP Weights (Analytic Hierarchy Process) Yalcin, A. (2008). GIS-based landslide susceptibility mapping using analytical hierarchy process and bivariate statistics in Ardesen (Turkey). Catena, 72(1), 1–12. Pourghasemi, H.R. et al. (2012). Application of fuzzy logic and analytical hierarchy process (AHP) to landslide susceptibility mapping at Haraz watershed, Iran. Natural Hazards, 63, 965–996.

- MCE — Multi-Criteria Evaluation Pradhan, B. & Lee, S. (2010). Regional landslide susceptibility analysis using back-propagation neural network model at Cameron Highland, Malaysia. Landslides, 7(1), 13–30. Yalcin, A. (2008). Catena, 72(1), 1–12.

- SINMAP Factor of Safety (FS) Pack, R.T., Tarboton, D.G. & Goodwin, C.N. (1998). The SINMAP approach to terrain stability mapping. 8th Congress of the International Association of Engineering Geology, Vancouver, Canada. FS thresholds used: <1.0, 1.0–1.25, 1.25–1.5, 1.5–2.0, >2.0

- TWI — Topographic Wetness Index Beven, K.J. & Kirkby, M.J. (1979). A physically based, variable contributing area model of basin hydrology. Hydrological Sciences Bulletin, 24(1), 43–69. Also applied in: Bui, D.T. et al. (2016). Remote Sensing, 8(11), 901.

- TRI — Terrain Ruggedness Index Riley, S.J., DeGloria, S.D. & Elliot, R. (1999). A terrain ruggedness index that quantifies topographic heterogeneity. Intermountain Journal of Sciences, 5(1–4), 23–27.

- SPI — Stream Power Index Moore, I.D., Grayson, R.B. & Ladson, A.R. (1991). Digital terrain modelling: A review of hydrological, geomorphological, and biological applications. Hydrological Processes, 5(1), 3–30.

- Plan & Profile Curvature Zevenbergen, L.W. & Thorne, C.R. (1987). Quantitative analysis of land surface topography. Earth Surface Processes and Landforms, 12(1), 47–56.

- Distance to Fault Yalcin, A. (2008). Catena, 72(1), 1–12. Fixed thresholds: <500 m, 500–1000 m, 1–2 km, 2–5 km, >5 km

- Drainage Density Horton, R.E. (1945). Erosional development of streams and their drainage basins: Hydrophysical approach to quantitative morphology. Bulletin of the Geological Society of America, 56(3), 275–370. Applied in susceptibility context: Hong, H. et al. (2017). Science of the Total Environment, 582, 756–769.

- HSG — Hydrologic Soil Group USDA Natural Resources Conservation Service (2009). National Engineering Handbook, Part 630: Hydrology. United States Department of Agriculture.

- LULC — Land Use / Land Cover & Root Cohesion Greenway, D.R. (1987). Vegetation and slope stability. In M.G. Anderson & K.S. Richards (Eds.), Slope Stability, pp. 187–230. Wiley, New York. Scores used: Forest = 1 (lowest risk), Bare land = 5 (highest risk)

- Adaptive Percentile Thresholds (P20/P40/P60/P80) Bui, D.T. et al. (2016). Landslide susceptibility assessment in the Hoa Binh province of Vietnam using frequency ratio and logistic regression. Remote Sensing, 8(11), 901. Hong, H. et al. (2017). Landslide susceptibility mapping using logistic regression and random forest. Science of the Total Environment, 582, 756–769.

- MFD — Multiple Flow Direction Quinn, P., Beven, K., Chevallier, P. & Planchon, O. (1991). The prediction of hillslope flow paths for distributed hydrological modelling using digital terrain models. Hydrological Processes, 5(1), 59–79.

- DEM / SRTM Data Farr, T.G. et al. (2007). The Shuttle Radar Topography Mission. Reviews of Geophysics, 45(2), RG2004. doi:10.1029/2005RG000183

- Depression Filling (Pit Removal) Wang, L. & Liu, H. (2006). An efficient method for identifying and filling surface depressions in digital elevation models for hydrologic analysis and modelling. International Journal of Geographical Information Science, 20(2), 193–213.

- Infinite Slope Stability Model (underlying FS formula) Skempton, A.W. & DeLory, F.A. (1957). Stability of natural slopes in London clay. Proceedings of the 4th International Conference on Soil Mechanics and Foundation Engineering, 2, 378–381.